SIMATS ENGINEERING

Department of Computer Science & Engineering

ASSESSMENT TOOL 2-DEBUGGING & OPTIMISATION TASK

| Course Code: | CSA12 | Course Title: | Computer Architecture |
| --- | --- | --- | --- |
| CO Assessed: | CO2 | Assessment: | ASSESSMENT TOOL3DEBUGGING & OPTIMISATION TASK |
| Weightage: | 20% | Total Marks: | 25 |
| CO2 Statement: | Apply integer and floating-point arithmetic algorithms including Booth's multiplication, restoring/non-restoring division, and IEEE 754 standard operations to solve fixed-point and floating-point computational problems. (BL3) |  |  |
| Student Name: | DIVYA SHREE.K | Reg.No.: | 192521376 |
| Department: | B.TECH-IT | Date: | 01/08/2026 |

ANNEXURE A

Code of Conduct:

I DIVYA SHREE.K(192521376) (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

DIVYA SHREE.K

MARKS ALLOCATION

|  | Identification of Errors / Bugs(20%) | Debugging Approach & Analysis(20%) | Correctnes s of Debugged Solution(2 5%) | Optimization Techniques Applied(20%) | Testing and Documentation(15 %) |
| --- | --- | --- | --- | --- | --- |
| Q1 |  |  |  |  |  |
| Q2 |  |  |  |  |  |
| Q3 |  |  |  |  |  |
| Q4 |  |  |  |  |  |
| Q5 |  |  |  |  |  |

RUBRICS FOR CALCULATION:

| Criteria | Weightage | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Identification of Errors / Bugs | 20% | Accurately identifies all errors and issues with complete understanding of the problem | Identifies most errors with minor omissions | Identifies limited errors; misses key issues | Unable to identify errors or misunderstands the problem |
| Debugging Approach & Analysis | 20% | Uses effective debugging techniques with clear analysis and root cause identification | Applies suitable debugging methods with minor gaps | Uses basic debugging methods with limited analysis | No systematic debugging approach followed |
| Correctness of Debugged Solution | 25% | Provides completely corrected solution with accurate functionality and expected output | Provides working solution with minor errors | Partially correct solution with limited functionality | Incorrect solution or fails to resolve errors |
| Optimization Techniques Applied | 20% | Applies efficient optimization methods to improve performance, memory usage and quality | Performs optimization with noticeable improvements | Limited optimization with minimal improvements | No optimization applied or inefficient implementation |
| Testing and Documentation | 15% | Performs complete testing with proper validation and clear documentation | Provides adequate testing and documentation with minor issues | Limited testing and incomplete documentation | No proper testing or documentation provided |

1.A program performing signed integer addition using 2’s complement produces incorrect results when adding large values (e.g., +120 and +50 in 8-bit representation). Debug the issue by analyzing the binary computation and identifying whether overflow has occurred. Propose a solution to correctly detect and handle overflow in such cases.

Debugging 2's Complement Addition Overflow

Problem:
The program adds +120 and +50 using 8-bit signed integers.

Binary Representation:

- +120 = 01111000

- +50 = 00110010

Addition:

01111000

+ 00110010

------------

10101010

Result Analysis:

- Binary result = 10101010

- In 8-bit 2's complement, this represents -86.

- But the correct answer is 170, which cannot be represented in an 8-bit signed integer (-128 to +127).

Issue:

- Overflow has occurred.

- Overflow happens when two positive numbers produce a negative result.

Solution:

- Detect overflow by checking:

  - If both operands have the same sign and the result has a different sign.

- Use a larger data type (16-bit or 32-bit) or display an overflow error.

2.A processor using a ripple carry adder shows significant delay in executing arithmetic operations in a real-time system. Debug the performance bottleneck by examining how carry propagates through each bit. Suggest an optimized solution using a carry lookahead adder, and explain how it reduces delay.

Ripple Carry Adder Delay and Carry Lookahead Adder

Problem:
       A ripple carry adder is slow because each bit must wait for the carry from         the previous bit.

Cause of Delay:

- Carry propagates one bit at a time.

- For an n-bit adder, the delay increases with the number of bits.

Example:

Bit0 → Bit1 → Bit2 → Bit3 → ... → Bit7

Each stage waits for the previous carry.

Optimized Solution: Carry Lookahead Adder (CLA)

- CLA calculates carry bits in parallel using Generate (G) and Propagate (P) signals.

- Carries are produced without waiting for previous stages.

Advantages:

- Faster addition.

- Reduced propagation delay.

- Suitable for real-time systems.

3. A multiplication unit implementing Booth’s algorithm gives incorrect results for certain negative operands. Debug the step-by-step execution of the algorithm, focusing on bitpair checking, arithmetic shifts, and sign extension. Identify the possible source of error and suggest corrections to ensure accurate signed multiplication.

Debugging Booth's Multiplication Algorithm

Problem:
Incorrect results occur when multiplying negative numbers.

Possible Errors:

- Incorrect bit-pair checking.

- Wrong arithmetic right shift.

- Improper sign extension.

Correct Workflow:

- Check the last two bits (Q₀ and Q₋₁).

- Perform:

  - 01 → Add multiplicand

  - 10 → Subtract multiplicand

  - 00 or 11 → No operation

- Perform arithmetic right shift.

- Preserve the sign bit during shifting.

Cause of Error:

- Logical shift instead of arithmetic shift.

- Incorrect sign extension.

- Wrong bit-pair interpretation.

Solution:

- Use arithmetic right shifts.

- Maintain sign extension.

- Verify bit-pair logic in every iteration.

4.An embedded system using the restoring division algorithm experiences inefficiency due to repeated restoration steps, leading to increased execution time. Debug the algorithm’s workflow and identify where unnecessary operations occur. Propose an optimized approach using the non-restoring division method, explaining how it improves performance.

Restoring Division vs Non-Restoring Division

Problem:
The restoring division algorithm is slow because it repeatedly restores the previous value after unsuccessful subtraction.

Workflow:

- Shift left.

- Subtract divisor.

- If result is negative:

  - Restore previous value.

- Repeat.

Issue:

- Frequent restoration increases execution time.

Optimized Solution: Non-Restoring Division

- Does not restore immediately.

- Performs addition or subtraction in the next step depending on the previous result.

Advantages:

- Fewer operations.

- Faster execution.

- Better efficiency in embedded systems.

- A scientific application using IEEE 754 single precision floating-point representation produces inaccurate results when adding very small and very large numbers. Debug the issue by analyzing exponent alignment, normalization, and rounding errors. Suggest optimization techniques, such as using double precision or algorithmic adjustments, to improve accuracy.

. IEEE 754 Floating-Point Accuracy Problem

Problem:
Adding a very small number to a very large number gives inaccurate results.

Example:

1,000,000.0 + 0.000001

The small value may disappear after exponent alignment.

Reason:

- Exponents are aligned before addition.

- The smaller number loses significant bits.

- Rounding causes precision loss.

- Normalization may remove tiny values.

Solution:

- Use double precision (64-bit) instead of single precision (32-bit).

- Use numerically stable algorithms (such as compensated summation/Kahan summation) to reduce rounding error.

- Avoid repeatedly adding very small values to very large values when possible.

Advantages:

- Higher precision.

- Reduced rounding errors.

- More accurate scientific calculations.